QR Code-Based Inventory Management System

1. Components and Their Roles

(a) Raspberry Pi 4B (4GB RAM)

- Role: Main processing unit for QR code scanning, data processing, and integration with Google Sheets.

- Why Used?: Runs Python scripts and supports USB peripherals.

(b) MH-ET LIVE Scanner (QR Code Scanner)

- Role: Reads QR codes and sends data to Raspberry Pi.

- Why Used?: Outputs data via USB (keyboard emulation) or I2C communication.

(c) SD Card (at least 16GB recommended)

- Role: Stores Raspberry Pi OS, Python scripts, and dependencies.

(d) Power Cable

- Role: Provides power to Raspberry Pi.

- Why Used?: Requires 5V, 3A USB-C power supply.

(e) Jumper Wires (Male-Female)

- Role: Connects Raspberry Pi and other components (if I2C is used).

(f) Resistors (4.7kΩ x2)

- Role: Used for pull-up resistors in I2C communication.

2. Hardware Connections

Method 1: USB Connection (Preferred)

- Plug the MH-ET LIVE scanner into any USB port on the Raspberry Pi.

- Acts like a keyboard, inputting scanned QR codes as text.

Method 2: I2C Connection (Alternative)

| QR Scanner Pin | Raspberry Pi GPIO Pin |
| --- | --- |
| SDA | GPIO 2 (Pin 3) |
| SCL | GPIO 3 (Pin 5) |
| GND | GND (Pin 6) |
| VCC (5V) | 5V (Pin 2) |

Enable I2C on Raspberry Pi:

# Navigate to Interfacing Options -> Enable I2C

3. Software Implementation

Step 1: Setting Up the Raspberry Pi

- Install Raspberry Pi OS.

- Enable I2C in Raspberry Pi settings (raspi-config).

- Install required packages:

- sudo apt update && sudo apt install python3-pip i2c-tools

Step 2: Installing Python Libraries

- smbus2: For I2C communication.

- gspread: For Google Sheets integration.

- pandas: For data handling.

Step 3: Reading QR Code Data via I2C

4. Google Sheets Integration

Step 1: Enable Google Sheets API

- Enable Google Sheets API in Google Cloud Console.

- Create a service account and download the credentials JSON file.

- Share the Google Sheet with the service account email.

Step 2: Python Script to Update Google Sheets

5. Categorizing Inventory Items

Google Sheets Structure

| QR Code | Item Name | Date & Time | Stock Quantity |
| --- | --- | --- | --- |
| ABC123 | Mouse | 2025-02-26 14:30 | 10 |
| XYZ456 | Keyboard | 2025-02-26 14:35 | 5 |

Updated Python Script for Categorization

6. Final Project Workflow

- Power on Raspberry Pi.

- Run the QR scanner script (USB or I2C method).

- Scan a QR code.

- Extract data and send it to Google Sheets.

- Verify inventory update in Google Sheets.